1)FSV AND ITS TYPES:

FSP (Firmware Support ) is an Intel-provided binary firmware component that initializes silicon hardware (CPU, chipset, memory) during the early boot stages.

FSP executes the low-level CPU instructions that directly program hardware registers to make that hardware usable.

Why FSP exists

Modern CPUs and chipsets are extremely complex. Intel provides FSP so that:

- Board/BIOS developers don’t need to write low-level silicon init code

- Hardware initialization is consistent and validated

- Time-to-market is faster

FSP is split into three logical parts:

1️⃣ FSP-T (Temporary RAM Init)

- Sets up Cache-as-RAM

- Allows PEI phase to run

- Very first FSP entry point

2️⃣ FSP-M (Memory Init)

- Initializes DRAM

- Trains memory

- Hands over real RAM to BIOS

3️⃣ FSP-S (Silicon Init)

- Initializes remaining silicon:

  - PCIe

  - USB

  - Power management

  - SoC features

2. what is boot policy?

Boot Policy means the rules the firmware follows to decide how, from where, and in what order the system boots an OS. It is stored in BIOS/UEFI variables (NVRAM)and implemented by DXE + BDS (Boot Device Selection) phase.

1️⃣ Boot device order

- SSD first, then USB, then Network

- Legacy vs UEFI boot

- Fallback if first device fails

2️⃣ Boot mode rules

- UEFI boot only

- Legacy (CSM) allowed or not

- Mixed mode allowed

3️⃣ Security rules

- Secure Boot enabled/disabled

- Only signed OS loaders allowed

- Block unknown USB boot

3. diff b/w text mode and graphics mode

Text Mode:

- Displays characters only (letters, numbers, symbols)

- Screen is divided into rows and columns (like 80×25)

- Each screen position holds one character

- Uses very little memory

- Faster to display

- Examples: Command Prompt, DOS screen, terminal

Best for: typing commands, programming, simple output

Graphics Mode:

- Displays images, shapes, colours, and text

- Screen is divided into pixels

- Each pixel can have a different colour

- Uses more memory

- Slower compared to text mode

- Examples: Windows desktop, games, drawing software

Best for: pictures, videos, games, graphical user interfaces (GUI)

4. what’s ACPI?

ACPI stands for Advanced Configuration and Power Interface.

ACPI is a standard that tells the operating system how to manage power and hardware using information provided by the BIOS/UEFI.

Why ACPI exists

Before ACPI:

- BIOS controlled power

- OS had little control

With ACPI:

- OS controls power

- BIOS just describes the hardware and rules

So:

BIOS describes → OS decides → Hardware obeys

What ACPI does

1️⃣ Power management

- Sleep states

- Shutdown

- Idle power saving

2️⃣ Device power control

- Turn devices ON/OFF

- Save power when idle

3️⃣ Hardware description

- Tells OS:

  - Which devices exist

  - How they are connected

  - How to control them

ACPI power states (important)

| System states State | Meaning |
| --- | --- |
| S0 | System ON |
| S3 | Sleep (Suspend to RAM) |
| S4 | Hibernate |
| S5 | Soft OFF |

Device states

- D0 → Fully ON

- D3 → OFF

5.what is a software hook?

A software hook is a mechanism that allows one piece of software to intercept, monitor, modify, or extend the behaviour of another piece of software at a specific point.

How a software hook works

At a defined point in execution:

- Program reaches a hook point

- Control is transferred to hook code

- Hook code runs (log, modify, block, extend)

- Control returns to original flow

In firmware:

A hook is a predefined callback that platform code can register to run at a specific boot or runtime stage.

Examples:

- Before memory init

- Before OS handoff

- On S3 resume

- On power button press

6.What is SMM?

System Management Mode (SMM) is a special, ultra-privileged CPU mode on x86 processors, designed for low-level system management tasks that must be:

- Invisible to the OS

- Isolated from normal software

- Protected from malware

It was introduced by Intel for power and hardware management, but today it is the most security-critical execution environment in firmware.

SMM is entered only when an SMI occurs.

When an SMI fires:

- CPU immediately stops current execution

- CPU state is automatically saved

- CPU jumps to SMM code

- OS has zero awareness

CPU State Is Saved To: SMRAM

- All registers

- Control registers

- Segment registers

- Instruction pointer

This saved context is called the SMI Save State.

7.What is CRC (Cyclic Redundancy Check)?

CRC (Cyclic Redundancy Check) is an error-detection technique used to verify the integrity of data—to check whether data was altered, corrupted, or damaged during:

- Storage (flash, disk, memory)

- Transmission (USB, PCIe, Ethernet)

- Firmware updates (BIOS/UEFI images)

CRC detects errors; it does NOT correct them.

Where CRC Is Used

- Firmware Volume (FV) headers

- Capsule update validation

- NVRAM variable integrity

- Option ROM validation

8.What is a Patch File?

A patch file is a text file that describes changes to be applied to one or more source files without sending the full files again.

It contains:

- What lines were added

- What lines were removed

- Where the change applies

Patch files are heavily used in BIOS/UEFI, Linux kernel, firmware, and large codebases

This makes:

- Code review easier

- Version control cleaner

- Collaboration safer

- Integration faster

9.What is a Makefile?

A Makefile is a build instruction file used by the tool make (common on Linux/Unix).

It tells the build system:

- What to build

- How to build it

- In what order

- When to rebuild

Why Makefiles Exist

Large projects have:

- Hundreds of source files

- Dependencies between files

- Different build configurations

Makefile:

- Rebuilds only what changed

- Saves time

- Avoids human errors

What is an NMake File?

An NMake file serves the same purpose as a Makefile but:

- Uses Microsoft syntax

- Uses Visual C++ compiler (cl.exe)

In EDK2:

- You do NOT write Makefiles

- You write:

  - .INF (module description)

  - .DSC (platform description)

  - .FDF (flash layout)

EDK2 internally:

- Generates Makefiles / NMake files

- Uses nmake on Windows

- Uses make on Linux

11.What is FSP?

FSP (Firmware Support Package) is a binary-only silicon initialization package provided by Intel.

It performs early CPU + chipset initialization, so OEMs don’t need to write or maintain complex silicon bring-up code.

Think of FSP as:
“Intel initializes the silicon, BIOS focuses on platform policy.”

What is FSP 2.0?

FSP 2.0 is the modern, standardized version of Intel FSP used in UEFI-compliant platforms.

It cleanly separates silicon initialization from platform firmware.

What FSP 2.0 Does

FSP 2.0 initializes:

- CPU cores

- Memory (DDR training)

- Chipset fabric

- Power management

- I/O controllers (basic setup)

And then returns control back to the BIOS.

FSP 2.0 Architecture

FSP 2.0 is divided into three binary components:

FSP-T (TempRamInit)

- Runs very early

- Sets up temporary RAM (Cache-as-RAM)

- No DRAM yet

Used in SEC / early PEI

FSP-M (MemoryInit)

- Initializes DRAM

- Performs memory training

- Configures memory controller

This is the most critical part

FSP-S (SiliconInit)

- Initializes remaining silicon

- PCIe root ports

- Integrated graphics

FSP-X is an extended / next-generation FSP model used in newer Intel platforms, especially those targeting:

- Client SoCs

- Hybrid architectures

- Modular firmware stacks

FSP-X is not a replacement — it is an evolution.

FSP-X Philosophy

FSP-X is designed for:

- Complex SoCs

- Multiple power domains

- Fine-grained silicon control

- Faster resume paths

12. What is an Option ROM?

An Option ROM is firmware code stored on a hardware device (or bundled with system firmware) that allows the BIOS/UEFI to recognize, initialize, and optionally boot from that device before the OS loads.

Why Option ROM Exists

When a system powers on:

- BIOS knows CPU, RAM, chipset

- But does NOT know how every add-in card works

Option ROM provides:

- Initialization logic

- Boot services

- Legacy compatibility

Where Option ROM Lives

Option ROM can be stored in:

- Flash on the device (NIC, RAID card, GPU)

- System firmware flash (integrated devices)

- Embedded UEFI driver inside BIOS

Examples:

- Network card PXE ROM

- RAID controller ROM

- GPU VBIOS

13. What is GPIO?

GPIO (General Purpose Input/Output) is a programmable digital pin on a chipset / SoC / microcontroller that firmware or software can control to:

- Read a signal (input)

- Drive a signal (output)

In simple terms:
GPIO = a software-controlled on/off pin

Why GPIO Exists

GPIOs are used to connect the CPU/chipset to external hardware signals that are not complex enough to need full controllers.

Common uses:

- Power button

- Reset signals

- LEDs

- Fan enable

- Board ID straps

- Detect lid open/close

- Enable peripherals (Wi-Fi, BT, camera)

14. What IA-32 and x64 Mean

IA-32

- 32-bit x86 architecture

- Introduced with Intel 80386

- Commonly called:

  - x86

  - 32-bit mode

- Used in older PCs, legacy OS, early BIOS

x64 (x86-64 / AMD64)

- 64-bit extension of x86

- Introduced by AMD, later adopted by Intel

- Used in modern CPUs, UEFI, modern OS

x64 is backward compatible with IA-32.

15. Difference between assembly language and machine language?

What is Machine Language?

Machine language is the raw binary instructions executed directly by the CPU.

- Consists of 0s and 1s

- CPU executes it without translation

- Different for IA32, X64, ARM, etc.

- Humans cannot easily read or write this.

What is Assembly Language?

Assembly language is a symbolic, human-readable representation of machine instructions.

- Uses mnemonics (MOV, ADD, JMP)

- Still CPU-specific

- Needs an assembler to convert it to machine code

## Relationship Between Them

Assembly Code

↓ (Assembler)

Machine Code

↓

CPU executes

CPU NEVER executes assembly directly

16.What is SPI?

SPI (Serial Peripheral Interface):

SPI is a synchronous serial communication protocol used to communicate between:

- CPU (master)

- External devices (slaves) like flash, TPM, sensors

Key point:

In BIOS systems, SPI is mainly used to access SPI Flash (where BIOS is stored).

What is Flash (SPI Flash)?

Flash Memory (in BIOS context)

SPI Flash is non-volatile memory that stores:

- BIOS / UEFI firmware

- Boot code

- Microcode

- NVRAM variables

- Data remains even when power is OFF.

Why Flash is needed

RAM:

- Volatile

- Empty after reset

Flash:

- Non-volatile

- CPU fetches first instruction from flash

17.What are Device Parameters?

Device Parameters = Configuration values that define how the SPI flash behaves

These parameters tell the controller:

- How fast it can run

- How data is erased

- How writes work

- Size and layout

Common SPI Flash Device Parameters

Flash Size

- Example: 8 MB, 16 MB, 32 MB

- Defines address width (24-bit / 32-bit)

Page Size

- Smallest writable unit

- Usually 256 bytes

You cannot write single bytes directly.

Sector Size

- Smallest erasable block

- Common values:

  - 4 KB

  - 64 KB

Erase before write is mandatory

Erase Commands

Different erase granularities:

| Erase Type | Size |
| --- | --- |
| Sector Erase | 4 KB |
| Block Erase | 64 KB |
| Chip Erase | Entire flash |

How BIOS Uses Device Parameters

During early boot:

- SPI controller initializes

- Reads JEDEC ID

- Matches flash model

- Loads device parameter table

- Sets:

  - Clock

  - Read mode

  - Addressing mode

18. What is Erase Polarity?

Erase polarity defines what bit value (0 or 1) a flash memory cell has after an erase operation.

Erase polarity = the logical value of flash bits after erase

It answers this exact question:

After I erase flash memory, are the bits 1 or 0?

19.What is LZMA Compression?

LZMA (Lempel-Ziv-Markov chain Algorithm) is a high-ratio, lossless compression algorithm widely used in BIOS/UEFI firmware, bootloaders, and embedded systems.

Lossless → original data is recovered exactly
High compression → saves a lot of flash space

Why LZMA Exists

Firmware storage (SPI flash) is very limited (8–32 MB).
LZMA allows:

- Smaller firmware images

- More features in the same flash

- Faster boot reads (less data to fetch)

How LZMA Works

LZMA combines two techniques:

LZ (Dictionary Compression)

- Finds repeated byte sequences

- Replaces them with references (distance + length)

Markov Chain + Range Coding

- Predicts probability of next bits

- Encodes data efficiently using range encoding

This is why LZMA compresses better than ZIP or GZIP, but needs more CPU.

20. What is flash device?

Definition

A flash device is a non-volatile storage device that stores data using flash memory technology.
It retains data even when power is turned OFF and does not contain any moving parts.

What is Flash Memory?

Flash memory is a type of electronic memory that stores data in memory cells using electrical charges.
Because it is non-volatile, the stored data is not lost when power is removed.

21. Difference between flash device and storage device?

Definition

A storage device is a hardware component used to store data, programs, and information permanently or temporarily in a computer system.

Purpose of Storage Devices

- Store operating systems

- Store applications and user data

- Backup and archive information

- Transfer data between systems

Types of Storage Devices

Storage devices can be classified based on the technology used:

- Magnetic Storage

  - Hard Disk Drive (HDD)

  - Magnetic Tape

- Optical Storage

  - CD

  - DVD

- Solid-State Storage

  - Flash devices (USB, SSD, SD card)

22. Flash device vendors list?

Flash Device Vendors

Flash device vendors are companies that manufacture flash memory or flash-based storage products such as SSDs, USB drives, memory cards, and embedded flash.

Major Flash Memory Manufacturers (Chip Makers)

These companies design and manufacture flash memory chips used worldwide:

- Samsung Electronics

- Micron Technology

- Intel

23. PCI vendors id and device id list?

What is PCI Vendor ID?

A PCI Vendor ID is a 16-bit hexadecimal number assigned by the PCI-SIG to uniquely identify a hardware manufacturer.

- Written in hexadecimal (e.g., 0x8086)

- Identifies who made the device

- Same Vendor ID is used for all devices from that company

What is PCI Device ID?

A PCI Device ID is a 16-bit hexadecimal number assigned by the vendor to identify a specific device or chipset.

- Written in hexadecimal (e.g., 0x100E)

- Identifies which device from that vendor

- Combined with Vendor ID to uniquely identify hardware

# How Vendor ID and Device ID Work Together

- Vendor ID = Who made the device

- Device ID = Which device it is

Example:

- Vendor ID: 0x8086 → Intel

- Device ID: 0x100E → Intel Gigabit NIC

24. What is device path in UEFI?

Definition

A Device Path in UEFI is a structured data representation that uniquely describes the location of a device or file in the system firmware environment.

It tells UEFI how to reach a device step-by-step, starting from the root of the system hardware.

Why Device Path Is Needed

UEFI supports many device types (PCI, USB, SATA, NVMe, filesystems, network).
A single identifier like a number is not enough.

Device Path allows UEFI to:

- Locate hardware devices

- Identify boot devices

- Load drivers

- Locate files (EFI applications)

25. What are nested macros?

Definition

Nested macros are macros defined or used inside another macro.
In other words, one macro expands to code that contains another macro, which is then expanded by the preprocessor

Advantages of Nested Macros

- Code reuse

- Centralized changes

- Shorter and cleaner code

- Useful in low-level programming (BIOS, UEFI, drivers)

Disadvantages of Nested Macros

- Difficult to debug

- Harder to read if overused

- Can cause unexpected behaviour if not carefully written

26.What are mmio and page tables?

1. Memory-Mapped I/O (MMIO)

Definition

Memory-Mapped I/O (MMIO) is a mechanism where hardware device registers are mapped into the system’s physical memory address space. The CPU accesses these device registers using the same load and store instructions used for normal memory.

Purpose of MMIO

MMIO allows:

- Uniform access to devices using standard memory instructions

- Simplified CPU and compiler design

- Faster device access compared to legacy I/O methods

- Compatibility with modern 32-bit and 64-bit architectures

How MMIO Works

- A hardware device exposes control and status registers at fixed physical addresses.

- The operating system or firmware maps these addresses into its address space.

- Software reads or writes to these addresses.

- The hardware device responds to the access.

## Page Tables

### Definition: Page tables are data structures used by the CPU’s Memory Management Unit (MMU) to translate virtual addresses used by software into physical addresses used by hardware.

### Why Page Tables Exist

### Programs use virtual addresses
RAM uses physical addresses

### The page table connects the two.

### Virtual Address ──▶ Page Table ──▶ Physical Address

### Purpose of Page Tables

### Page tables provide:

- Memory isolation between processes

- Protection and access control

- Support for virtual memory

### 27.What is HID and UID?

### 1. HID (Hardware ID)

### Definition

### HID (Hardware ID) is an identifier used by firmware and operating systems to identify the type or class of a hardware device. It tells the system what the device is, not which exact physical unit it is.

### HID is commonly used during:

- Device enumeration

- Driver matching

- Hardware classification

### Purpose of HID

### HID is used to:

- Identify a device’s function or category

- Select the correct driver

- Distinguish between different device types

## UID (Unique ID)

### Definition

UID (Unique ID) is an identifier that uniquely distinguishes one physical device from another, even if both devices are of the same model.

Purpose of UID

UID is used to:

- Uniquely identify hardware instances

- Track devices across reboots

- Bind configuration or state to a specific device

- Support security and licensing mechanisms

How HID and UID Work Together

When a device is detected:

- HID identifies what kind of device it is.

- UID identifies which exact device it is.

- The system:

  - Loads the correct driver using HID

  - Applies per-device settings using UID

28.What is micro code?

Microcode

1. Definition

Microcode is a low-level set of internal instructions stored inside a CPU that controls how machine-level instructions are executed by the processor.

It acts as an intermediate layer between:

- Machine instructions (x86, ARM, etc.)

- Physical CPU hardware (ALU, registers, pipelines)

Why Microcode Exists

Modern CPU instructions are complex and cannot always be executed directly by hardware in a single step. Microcode breaks these complex instructions into simpler internal operations (called micro-operations or µops).

Microcode allows:

- Simplification of CPU hardware design

- Support for complex instruction sets (e.g., x86)

- Bug fixes after manufacturing

- Behavioural consistency across CPU generations

29. What are the cpu real modes?

1. CPU Real Modes

Definition

CPU modes define how the processor operates and what features are available.

Common CPU Modes (x86)

- Real Mode

- Default mode after CPU reset

- 16-bit addressing

- Maximum 1 MB memory access

- No memory protection

- Used by legacy BIOS

- Protected Mode

  - 32-bit mode

  - Memory protection

  - Multitasking support

  - Paging support

  - Used by modern OS kernels

- Long Mode

  - 64-bit mode

  - Used by modern 64-bit OS

  - Supports large memory addressing

  - Includes compatibility mode for 32-bit apps

- System Management Mode (SMM)

  - Special hidden mode

  - Used for power and hardware management

  - Runs isolated from OS

30.Virtual memory and page table?

Virtual Memory

Virtual memory allows programs to use virtual addresses instead of physical addresses.

Benefits:

- Process isolation

- Memory protection

- Efficient memory usage

Page Tables

Page tables are data structures that map:

Virtual Address → Physical Address

They are used by the CPU’s Memory Management Unit (MMU).

Page tables also define:

- Access permissions (Read / Write / Execute)

- Memory type (RAM or MMIO)

- Cache behavior

31.What is notification and call back function?

Callback Function

A callback is a function passed as an argument and executed when a specific event occurs.

Example:

- Timer expiration

- Interrupt

- Button press

Notification

A notification is the mechanism that triggers a callback when an event happens.

Example:

- UEFI protocol notify

- OS event notification

32.What is s3 resume?

Definition

S3 Resume refers to the process of restoring a system from the S3 power state, also known as Suspend-to-RAM.

In the S3 state:

- System RAM remains powered

- CPU, chipset, and most devices are powered off

- System context is preserved in memory

S3 Resume Process

- System enters S3 state on sleep request

- RAM contents are preserved

- Power is removed from CPU and devices

- On wake-up event (keyboard, mouse, timer):

  - CPU resets

  - Firmware performs minimal initialization

  - Memory contents are restored

  - OS resumes execution from saved state

33.What is watch dog timer and monotonic counter?

A Watchdog Timer (WDT) is a hardware or software timer used to detect and recover from system failures.
It monitors whether the system (CPU / firmware / OS) is operating correctly.

If the system fails to respond or gets stuck (for example due to a software hang or infinite loop), the watchdog timer automatically resets or recovers the system.

Why is a Watchdog Timer Needed?

In real-time and low-level systems such as:

- BIOS / UEFI firmware

- Embedded systems

- Operating systems kernel

- Industrial controllers

there may be no user interaction to recover the system when it hangs.

The watchdog timer ensures:

- System reliability

- Automatic recovery

- Prevention of permanent system lock-ups

How Does a Watchdog Timer Work?

- The watchdog timer is started with a fixed timeout value

- The software must periodically “kick” or “pet” the watchdog

- If the software works correctly → watchdog keeps resetting

- If the software hangs or crashes → watchdog expires

- On expiry → system performs a reset, interrupt, or recovery action

34.what is FAT 32 and NTFS file system?

What is FAT32?

FAT32 (File Allocation Table 32) is an older and simple file system developed by Microsoft.
It is widely supported by almost all operating systems and devices.

Key Features of FAT32

- Uses a File Allocation Table to track file locations

- Simple structure

- Highly compatible across platforms

- No built-in security or permissions

FAT32 Limitations

- Maximum single file size: 4 GB

- Maximum partition size: ~2 TB

- No file permissions

- No encryption

- No journaling (less crash-safe)

What is NTFS?

NTFS (New Technology File System) is a modern, advanced file system developed by Microsoft for Windows operating systems.

Key Features of NTFS

- Supports very large files and partitions

- Journaling for crash recovery

- File permissions and access control

- File encryption and compression

- Better performance and reliability

NTFS Limits

- Maximum file size: Up to 16 TB (practically much larger)

- Maximum volume size: Up to 256 TB or more (depends on cluster size)

35.What is IHV and ODM and OEM?

An Independent Hardware Vendor (IHV) is a company that designs and manufactures hardware components which are sold to other companies for integration into larger systems.

Key Characteristics of IHV:

- Focuses on specific hardware components

- Supplies hardware to OEMs and ODMs

- Provides drivers, firmware, and technical support

- Hardware is usually sold under the IHV’s own brand

Role of IHV in BIOS / Firmware

- Provides reference code and specs

- Supplies firmware interfaces (ACPI, SMBIOS, MMIO specs)

- Works closely with BIOS vendors

ODM (Original Design Manufacturer)

What is ODM?

An Original Design Manufacturer (ODM) is a company that designs and manufactures a complete product which is later rebranded and sold by another company.

Key Characteristics of ODM

- Designs entire systems (motherboards, laptops, servers)

- Handles hardware design and manufacturing

- Product is sold under OEM’s brand

- May produce the same design for multiple brands

Role of ODM in BIOS / Firmware

- Integrates BIOS/UEFI

- Customizes firmware for platforms

- Works with IHVs for hardware compatibility

OEM (Original Equipment Manufacturer)

What is OEM?

An Original Equipment Manufacturer (OEM) is a company that sells the final product to customers under its own brand, even if it did not design or manufacture all components.

Key Characteristics of OEM

- Owns the brand and customer relationship

- Defines product requirements

- Handles marketing, sales, and support

- May or may not manufacture the product

Role of OEM in BIOS / Firmware

- Specifies firmware features

- Controls BIOS customization and branding

- Handles BIOS updates and customer support

36.What is DDR4 and DDR4LP?

DDR4 (Double Data Rate 4 SDRAM)

DDR4 is the fourth generation of Double Data Rate Synchronous Dynamic Random-Access Memory (SDRAM).

It is used as main system memory (RAM) in desktops, laptops, servers, and embedded platforms.

Key Features of DDR4:

- Transfers data on both rising and falling edges of the clock

- Higher speed than DDR3

- Lower power consumption than DDR3

- Higher memory density (supports larger RAM sizes)

Where DDR4 is Used?

- Desktop PCs

- Laptops

- Servers

- Workstations

DDR4LP (Low-Power DDR4)

DDR4LP stands for Low-Power DDR4.
It is a power-optimized version of DDR4, designed mainly for mobile and embedded platforms.

DDR4LP is also commonly referred to as LPDDR4 in industry usage.

Key Features of DDR4LP

- Lower operating voltage

- Reduced power consumption

- Optimized for battery-powered devices

Where DDR4LP is Used?

- Smartphones

- Tablets

- Embedded and IoT devices

37. USB interface descriptor, end point descriptor, config descriptor.

What are USB Descriptors?

USB descriptors are data structures that describe a USB device’s capabilities, configuration, and communication methods to the host (PC).

They are sent by the device to the host during USB enumeration.

1. USB Configuration Descriptor

What is a Configuration Descriptor?

A Configuration Descriptor describes one complete device configuration, including:

- Power requirements

- Number of interfaces

- All related interface and endpoint descriptors

A USB device can have multiple configurations, but only one configuration can be active at a time.

| Field | Description |
| --- | --- |

| bLength | Size of this descriptor |
| --- | --- |

| bDescriptorType | CONFIGURATION descriptor type |
| --- | --- |

| wTotalLength | Total size of all descriptors for this configuration |
| --- | --- |

Purpose

- Tells host how the device is powered

- Defines how many interfaces the device has

- Groups interfaces and endpoints together

### What is an Interface Descriptor?

An Interface Descriptor defines one functional unit of a USB device.

Each interface represents a logical function, such as:

- Keyboard

- Mouse

- Audio device

- Mass storage

Purpose

- Allows composite devices to expose multiple functions

- Used by OS to load the correct driver

### What is an Endpoint Descriptor?

- An Endpoint Descriptor defines a communication channel between host and device.

- Endpoints are unidirectional and used to transfer data.

| Type | Usage |
| --- | --- |
| Control | Device management (EP0) |
| Bulk | Large data transfer (USB storage) |
| Interrupt | Small, time-critical data (keyboard, mouse) |

Purpose

- Defines how data flows

- Specifies packet size and transfer timing

38.Difference between scan code and uni code?

1. Scan Code

What is a Scan Code?

A scan code is a hardware-level code generated by the keyboard when a key is pressed or released.

- Generated by the keyboard controller

- Represents the physical key location

- Independent of language or character set

Key Characteristics of Scan Code

- Generated on key press (make code) and key release (break code)

- Same scan code regardless of keyboard language

- Used by BIOS, firmware, and OS keyboard drivers

- Example: Pressing the A key generates a scan code, regardless of language.

What is Unicode?

Unicode is a universal character encoding standard that represents characters and symbols from all languages.

- Represents characters, not keys

- Language and layout dependent

- Used at application level

Key Characteristics of Unicode

- Can represent characters from all languages

- Same character has same Unicode value everywhere

- Generated after OS layout and language processing

- High-level representation

Example

- Character ‘A’ → Unicode U+0041

- Character ‘अ’ → Unicode U+0905

39.What is NV-RAM?

NV-RAM is a type of memory that retains stored data even when power is turned off, while still allowing random access like normal RAM.

In simple words:

NV-RAM combines the speed of RAM with the persistence of storage.

Why NV-RAM is Needed

Traditional RAM (DRAM):

- Fast

- ❌ Loses data when power is off

Traditional Storage (HDD/SSD):

- Non-volatile

- ❌ Slower than RAM

NV-RAM solves this gap by providing:

- Fast access

- Data persistence

Key Characteristics of NV-RAM

- Non-volatile (data preserved without power)

- Random access

- Faster than disks

- Used for critical system data

Common Types of NV-RAM

1. Flash Memory (Modern NV-RAM)

- EEPROM / NOR / NAND flash

- Used in:

  - BIOS / UEFI firmware

  - Embedded systems

Stores:

- Firmware code

- Configuration variables

2. NVRAM in UEFI

In UEFI systems:

- NVRAM is a reserved region of flash

- Stores:

  - Boot variables

  - Secure Boot keys

  - Boot order

  - Platform settings

Examples:

- BootOrder

- Boot####

- SecureBoot